SWINGO

(Shallow Water table INfiltration alGOrithm) is a Fortran command line program to calculate infiltration for soils bounded by a shallow water table.

Author(s) Name(s) and Affiliation:

Dr. Rafael Muñoz-Carpena, carpena@ufl.edu, Professor, Hydrology & Water Quality, Department of Agricultural & Biological Engineering, University of Florida, P.O. Box 110570, 287 Frazier Rogers Hall, Gainesville, FL  32611-0570, USA

Dr. Claire Lauvernet, Researcher, claire.lauvernet@irstea.fr, Irstea, UR MALY, centre de Lyon-Villeurbanne, 5 rue de la Doua-CS 70077, F-69626 Villeurbanne cedex, France.

Website

http://abe.ufl.edu/carpena/software/swingo.shtml

Description

SWINGO (Shallow Water table INfiltration alGOrithm) is a Fortran command line program to calculate infiltration for soils bounded by a shallow water table. SWINGO simulates infiltration in soils under realistic initially non-ponded conditions with a shallow water table and steady rainfall conditions. The program consists of a time-explicit infiltration solution based on a combination of approaches by Salvucci and Entekhabi (1995) and Chu (1997) with the assumption of a horizontal wetting front. It includes new integral formulae for calculation of the singular times (time of ponding, shift time and time to soil profile saturation). This stand-alone version of SWINGO was developed to study the effects of a shallow water table on infiltration (Muñoz-Carpena et al., 2017). The program is now coupled with VFSMOD to study the effects of a seasonal shallow water table on the effectiveness of vegetative filter strips for water quality mitigation (Lauvernet and Muñoz-Carpena, 2017).

Screen shots

Scientific articles

- Muñoz-Carpena, R., C. Lauvernet, N. Carluer. Shallow water table effects on water, sediment and pesticide transport in vegetative filter strips: Part A. Unsteady rainfall infiltration and soil water redistribution. Hydrol. Earth Syst. Sci. Discuss. in review, July 2017.

- C. Lauvernet and R. Muñoz-Carpena. Shallow water table effects on water, sediment and pesticide transport in vegetative filter strips: Part B. model coupling, application, factor importance and uncertainty. Hydrol. Earth Syst. Sci. Discuss. in review, July 2017.

Technical information

Operating system(s): Fortran 77 source code can be compiled cross-platform

Licence: The model and associated documentation is supplied as-is with no warranty - explicit or implied. The model is provided to you as an educational and research tool under the terms of the Open Source Initiative, Educational Community License, Version 2.0 http://opensource.org/licenses/ecl2.txt).

Output(s): The program input file (swingo.iso) is written in ASCII format and are compatible with VFSMOD. Outputs are printed to the screen during execution and can be piped to an output file. See details are provided on the web site.

Export format(s): N/A

Other information: